WL-1000CAM CGI commands V1.1

1. JPEG image commands

1.1 Get one JPEG image

Method: Get

URL: HTTP://camera_ip/jpg/image.jpg

Return: One JPEG image will be transferred to client.

1.2 Get one JPEG image without any authentication

Method: Get

URL: HTTP://camera_ip/loginfree.jpg

Return: One JPEG image will be transferred to client.

Note: User maybe change the loginfree filename.

1.3 Get one JPEG video stream

Method: Get

URL: HTTP://camera_ip/mjpg/video.mjpg

Return: One multipart JPEG video stream will be transferred to client.

Format:

HTTP/1.0 200 OK\r\n

Content-Type: multipart/x-mixed-replace;

boundary=myboundary\r\n

\r\n

--myboundary\r\n

Content-Type: image/jpeg\r\n

Content-Length: 15656\r\n

\r\n

<JPEG image data>\r\n

--myboundary\r\n

Content-Type: image/jpeg\r\n

Content-Length: 14978\r\n

\r\n

<JPEG image data>\r\n

--myboundary\r\n

Content-Type: image/jpeg\r\n

Content-Length: 15136\r\n

\r\n

<JPEG image data>\r\n

--myboundary\r\n

. . . . . .

2. System commands

2.1 Get system information and configuration

2.1.1 Get image properties supported by camera

Method: Get

URL: http://camera_ip/camera-cgi/admin/param.cgi?action=list

&group=Properties.Image

Return: Image resolution, rotation and format supported by camera

Example:

root.Properties.Image.Rotation=0

root.Properties.Image.Resolution=640x480,320x240,176x144

root.Properties.Image.Format=jpeg,mjpeg

Description: Camera doesn’t support rotation, supports 3 kinds of

resolution and supports JPEG format only.

2.1.2 Get current camera configuration of resolution and JPEG quality

Method: Get

URL: http://camera_ip/camera-cgi/admin/param.cgi?action=list

&group= Image.I0.Appearance

Return: Current configuration of resolution and JPEG quality

Example:

root.Image.I0.Appearance.Resolution=640x480

root.Image.I0.Appearance.Compression=0

Compression: value range is 0~100

0: Highest quality, large image size

100: Lowest quality, small image size

2.1.3 Get current FPS value of camera

Method: Get

URL: http://camera_ip/camera-cgi/admin/param.cgi?action=list

&group= Image.I0.Stream

Return: Current FPS value of camera (frame rate per second)

Example:

Image.I0.Stream.FPS=30

2.1.4 Get current configuration of camera sensor

Method: Get

URL: http://camera_ip/camera-cgi/admin/param.cgi?action=list

&group= ImageSource.I0.Sensor

Return: Current brightness and contrast 、Saturation、Hue value

Example:

ImageSource.I0.Sensor.Brightness=50

ImageSource.I0.Sensor.Contrast=50

ImageSource.I0.Sensor.Saturation=50

ImageSource.I0.Sensor.Hue=50

Brightness and Contrast 、Saturation、Hue value range is 0~99

2.1.5 Get default video format

Method: Get

URL: http://camera_ip/camera-cgi/admin/param.cgi?action=list

&group=Layout

Return: Camera default video format

Example:

Layout.DefaultVideoFormat=mjpeg

2.1.6 Get information about the brand

Method: Get

URL: http://camera_ip/camera-cgi/admin/param.cgi?action=list

&group=Brand

Return: Information about the brand and type of camera

Example:

root.Brand.Brand=OVISLINK

root.Brand.ProdFullName=OVISLINK WL-1000CAM Network Camera

root.Brand.ProdShortName=OVISLINK WL-1000CAM

root.Brand.ProdNbr=WL-1000CAM

root.Brand.ProdType=Network Camera

root.Brand.WebURL=http://www.airlive.com/

2.1.7 Get hardware MAC address of camera

Method: Get

URL: http://camera_ip/camera-cgi/admin/param.cgi?action=list

&group= Network.Interface.I0.Active.MACAddress

Return: Hardware MAC address of camera

Example:

Network.Interface.I0.Active.MACAddress=00:11:09:15:10:AB

2.1.8 Get Firmware Version, Model ID and Vendor ID

Method: Get

URL: http://camera_ip/camera-cgi/admin/param.cgi?action=list

&group=Properties.Firmware

Return: Firmware version, Model ID and Vendor ID of camera

Example:

Properties.Firmware.Version=v1.32

Properties.Firmware.ModelID=8

Properties.Firmware.VendorID=1

2.1.9 Get web port number

Method: Get

URL: http://camera_ip/camera-cgi/admin/param.cgi?action=list

&group= System.HTTPPort

Return: web port number

Example:

System.HTTPPort=80

2.1.10 Get private video port number

Method: Get

URL: http://camera_ip/camera-cgi/admin/param.cgi?action=list

&group= System.PrivateVideoPort

Return: web port number

Example:

System.PrivateVideoPort=4321

2.1.11 Get MaxUsers and CurrentUsers Number

Method: Get

URL: http://camera_ip/camera-cgi/admin/param.cgi?action=list

&group= Properties.VideoServer

Return: MaxUsers and CurrentUsers Number

Example:

Properties.VideoServer.MaxUsers=16

Properties.VideoServer.CurrentUsers=2

2.1.12 Get FTP Server

Method: Get

URL: http://camera_ip/camera-cgi/admin/param.cgi?action=list

&group=Network.FTP.Server

Return: FTP Server

Example:

Network.FTP.Server=192.168.2.166

2.1.13 Get FTP Username

Method: Get

URL: http://camera_ip/camera-cgi/admin/param.cgi?action=list

&group=Network.FTP.User

Return: FTP Username

Example:

Network.FTP.User=test

2.1.14 Get FTP Password

Method: Get

URL: http://camera_ip/camera-cgi/admin/param.cgi?action=list

&group=Network.FTP.Passwd

Return: FTP Password

Example:

Network.FTP.Passwd=test

2.1.15 Get FTP Path

Method: Get

URL: http://camera_ip/camera-cgi/admin/param.cgi?action=list

&group=Network.FTP.Path

Return: FTP Path

Example:

Network.FTP.Path=/

2.1.16 Get FTP Port

Method: Get

URL: http://camera_ip/camera-cgi/admin/param.cgi?action=list

&group=Network.FTP.Port

Return: FTP Port

Example:

Network.FTP.Port=21

2.2 Set system configuration

2.2.1: Set resolution of camera

Method: Get

URL: http://camera_ip/camera-cgi/admin/param.cgi?action=update

&Image.I0.Appearance.Resolution=Width X Height

Parameters: 640x480, 320x240, 176x144

Example: Set resolution to 320X240

http://camera_ip/camera-cgi/admin/param.cgi?action=update

&Image.I0.Appearance.Resolution=320x240

2.2.2: Set JPEG compression level

Method: Get

URL: http://camera_ip/camera-cgi/admin/param.cgi?action=update

&Image.I0.Appearance.Compression=Compression Level

Parameters: 0~100,

0: Highest quality, large image size

100: Lowest quality, small image size

Example: Set compression level to 90

http://camera_ip/camera-cgi/admin/param.cgi?action=update

&Image.I0.Appearance.Compression=90

2.2.3 Set camera FPS

Method: Get

URL: http://camera_ip/camera-cgi/admin/param.cgi?action=update

&Image.I0.Stream.FPS=FPS

Parameters: 1, 3, 5, 10, 15, 20, 25, 30

Example: Set FPS to 20

http://camera_ip/camera-cgi/admin/param.cgi?action=update

&Image.I0.Stream.FPS=20

2.2.4 Set brightness of camera sensor

Method: Get

URL: http://camera_ip/camera-cgi/admin/param.cgi?action=update

&ImageSource.I0.Sensor.Brightness=Brightness Level

Parameters: 0~99

Example: Set brightness to 60

http://camera_ip/camera-cgi/admin/param.cgi?action=update

&ImageSource.I0.Sensor.Brightness=60

2.2.5 Set contrast of camera sensor

Method: Get

URL: http://camera_ip/camera-cgi/admin/param.cgi?action=update

&ImageSource.I0.Sensor.Contrast=Contrast Level

Parameters: 0~99

Example: Set contrast to 60

http://camera_ip/camera-cgi/admin/param.cgi?action=update

&ImageSource.I0.Sensor.Contrast=60

2.2.6 Set saturation of camera sensor

Method: Get

URL: http://camera_ip/camera-cgi/admin/param.cgi?action=update

&ImageSource.I0.Sensor.Saturation=Saturation Level

Parameters: 0~99

Example: Set saturation to 50

http://camera_ip/camera-cgi/admin/param.cgi?action=update

&ImageSource.I0.Sensor.Saturation=50

2.2.7 Set hue of camera sensor

Method: Get

URL: http://camera_ip/camera-cgi/admin/param.cgi?action=update

&ImageSource.I0.Sensor.Hue=Hue Level

Parameters: 0~99

Example: Set hue to 50

http://camera_ip/camera-cgi/admin/param.cgi?action=update

&ImageSource.I0.Sensor.Hue=50

2.2.8 Set FTP Server

Method: Get

URL: http://camera_ip/camera-cgi/admin/param.cgi?action=update

&Network.FTP.Server=Server

Parameters: FTP Server

Example: Set FTP Server to 192.168.2.166

http://camera_ip/camera-cgi/admin/param.cgi?action=update

&Network.FTP.Server=192.168.2.166

2.2.9 Set FTP Username

Method: Get

URL: http://camera_ip/camera-cgi/admin/param.cgi?action=update

&Network.FTP.User=Username

Parameters: FTP Username

Example: Set FTP Username to test

http://camera_ip/camera-cgi/admin/param.cgi?action=update

&Network.FTP.User=test

2.2.10 Set FTP Password

Method: Get

URL: http://camera_ip/camera-cgi/admin/param.cgi?action=update

&Network.FTP.Passwd=Password

Parameters: FTP Password

Example: Set FTP Password to test

http://camera_ip/camera-cgi/admin/param.cgi?action=update

&Network.FTP.Passwd=test

2.2.11 Set FTP Path

Method: Get

URL: http://camera_ip/camera-cgi/admin/param.cgi?action=update

&Network.FTP.Path=Path

Parameters: FTP Path

Example: Set FTP Path to /

http://camera_ip/camera-cgi/admin/param.cgi?action=update

&Network.FTP.Path=/

2.2.12 Set FTP Port

Method: Get

URL: http://camera_ip/camera-cgi/admin/param.cgi?action=update

&Network.FTP.Port=Port

Parameters: FTP Port

Example: Set FTP Port to 21

http://camera_ip/camera-cgi/admin/param.cgi?action=update

&Network.FTP.Port=21